Controller	:	AVM ISDN Controller FRITZ!Card USB v2.0

Operating System	:	Windows XP/2000

Version	:	03.09.10

Build	:	02.04.22

Protocol	: 	DSS1

Instructions for Installing the AVM ISDN-Controller FRITZ!Card USB v2.0 for Microsoft Windows XP/2000 and DSS1

Note:	If the AVM ISDN-Controller FRITZ!Card USB v2.0 already has been installed in Windows XP/2000, simply call up the setup program from the diskette.

The AVM ISDN-Controller FRITZ!Card USB v2.0 is a device for personal computers with a universal serial bus (USB). Specially developed 32-bit drivers are supplied to enable operation in Microsoft Windows XP/2000. These true 32-bit device drivers provide complete support for all features of Microsoft Windows XP/2000.

Establish which D-channel protocol is provided for your ISDN line by the ISDN provider. These include the DSS1, NI-1 and 5ESS Custom protocols. Lines installed in Europe after 1994 are generally DSS1 lines (Euro-ISDN lines).

This controller installation is designed exclusively for DSS1 (Euro ISDN) lines.

Installing the Driver Software in Windows XP/2000

Windows XP/2000 recognizes the ISDN-Controller as a network adapter and lists it as such in the Device Manager.

After installation the CAPI 2.0 interface and the NDIS WAN interface are available in the entire system. This means that CAPI 2.0-based applications can be used in parallel operation with Remote Access Service (RAS) or Internet access via Microsoft Dial-up Networking.

The CAPI 2.0 included in the package supports both 16-bit and 32-bit applications. Applications in DOS windows are not supported.

Once the FRITZ!Card USB v2.0 has been connected to the computer, the Windows XP/2000 Add New Hardware Wizard is started automatically.

Insert the FRITZ!Card CD and follow the instructions on the screen. Note the following:

1.	The Add New Hardware Wizard recognizes a new Plug-and-Play device to be installed. Click "Next".

2.	In the following window, select the option "Search for the best driver for your device (recommended)" and confirm with "Next".

3.	Select the option "CD-ROM drive" in the next window and confirm with "Next".

4.	The Wizard announces that a driver was found. Click "Next" to install the driver.

5.	In the next dialog, Windows may instruct you to install software which is not digitally signed by Microsoft (depending on the driver signature options set on your computer). Answer such an inquiry with "Yes". Repeat this sequence in the next dialog for the AVM NDIS WAN CAPI Driver.

6.	When the message "Windows has finished installing the software that your new hardware device requires" appears, the "USB" LED on the ISDN-Controller lights up. If FRITZ!Card USB v2.0 is connected to a PBX extension, on some PBXs the LED labeled "D" for D channel lights up as well. Confirm by clicking the "Finish" button.

Upon completion of the installation of the driver software for the ISDN-Controller FRITZ!Card USB v2.0, an information window appears reporting that the FRITZ! communications software can be installed. If questions occur, please refer to the respective paragraph in the installation manual.

This concludes the installation of the drivers for the FRITZ!Card USB v2.0 delivered with Windows XP/2000. The controller is now ready for operation. Once installation is complete, CAPI 2.0 and the AVM NDIS WAN CAPI Driver are available.

Updating the drivers

1.	Start the "Setup.exe" file from the FRITZ!Card USB v2.0 driver CD. This file is located in the directory

<CD-ROM drive>:\WINDOWS.XP\

<CD-ROM drive>:\WINDOWS.2K\

or in the subdirectory of any drive where the driver update was saved.

2.	Now the welcome dialog of the installation program appears. (Press the "F1" key in any dialog to prompt the On-line Help and obtain additional information about the respective dialog. Click "Back" if you would like to return to the previous window from successive dialog window.)

Click "Next".

3.	In the next dialog, select "Update", the option for updating the drivers for the FRITZ!Card USB, and then "Next".

Note:	If controller software for a different line protocol (D-channel protocol) is already installed, the system will indicate that you are about to change the line protocol. You can abort here without making any changes to the system. If you confirm by clicking "Yes", the controller software for the DSS1 protocol (Euro ISDN) will be installed.

4.	In the next dialog you may be prompted to install software which is not digitally signed by Microsoft (depending on the driver signature options set on your system). Answer such an inquiry with "Yes". Repeat this procedure to install the AVM NDIS WAN CAPI Driver.

5.	In the final step, a dialog appears announcing that the driver software has been installed successfully. Click "Exit".

Once the installation program has concluded you may begin working with the ISDN-Controller immediately. All CAPI 2.0-based applications can be started. An AVM NDIS WAN CAPI Driver is also available in the system for RAS / Internet connections.

After a standard installation (see above), the FRITZ!Card Controller is always loaded automatically when the system is started. This connects the computer to ISDN and initializes the CAPI interface.

Removing the Controller FRITZ!Card USB v2.0

To remove the AVM ISDN-Controller FRITZ!Card USB v2.0 from Windows XP/2000, proceed as follows:

1.	Open the Windows XP/2000 Control Panel ("Start / Settings") and double-click the "Add/Remove Hardware" icon.

2.	The Add/Remove Hardware Wizard is started. Click "Next".

3.	In the next dialog, select the option "Uninstall/Unplug a device" and click "Next".

4.	Select the option "Uninstall a device". Confirm by clicking "Next".

5.	In the next window, select the entry "AVM ISDN-Controller FRITZ!Card USB v2.0" from the list and then click "Next".

6.	Confirm that the controller is to be removed by activating the option "Yes, I want to uninstall this device" and then clicking "Next".

7.	As a final step, click the "Finish" button.

The ISDN-Controller is removed from your system.

Special features of the FRITZ!Card USB v2.0

The COMMON-ISDN-API 2.0 driver supplied with the controller supports both Windows applications and (simultaneously) applications running in DOS windows. This CAPI driver can also be used with older applications designed for CAPI 1.1.

The controller software for the AVM passive ISDN-Controllers conforms to ISDN Driver Model Architecture (IDM Architecture). This software is an implementation of the CAPI 2.0 specification for AVM ISDN-Controllers.

Together with the current controller software, the AVM passive ISDN-Controllers support all ISDN services. The following B channel protocols are available: X.75, HDLC transparent, transparent, X.25, ISO 8208 (X.25 DTE-DTE), X.31 case a/b, T.70, T.90, Fax G3/4, V.110, V.120 and V.32bis.

CAPI SoftCompression X75/V42bis

Data compression during transmission is an additional feature available for data connections with X.75. The compression procedure used is V.42bis (CAPI SoftCompression X75/V42bis), in accordance with the CAPI 2.0 specification.

In the default settings for the controller software, automatic data compression is always available, but applications (e.g. FRITZ!data) must enable data compression directly to actually use it. You can use the FRITZ!Card USB v2.0 setup to permanently enable data compression at any time.

If you have enabled data compression, the remote sites negotiate the use of data compression during the connection setup. If a remote site does not have data compression, the data will be transmitted without using compression. This process is transparent for the user.

CAPI SoftFax G3 and CAPI SoftModem V32bis

In addition to the digital features, the FRITZ!Card USB v2.0 provides access to analog services such as Fax Group 3 (CAPI SoftFax G3) and Modem (Capi SoftModem V32).

With this controller software, the FRITZ!Card USB v2.0 can be used in Windows as a platform for any fax software. The controllers enable you to send and receive faxes of up to 14400 baud. In this case, both B channels are available for fax communication. The controller functions as a Group 3 fax machine.

The controller also incorporates a V.32bis soft modem. On the basis of this modem emulation, communications applications can be used to set up connections from an ISDN access to analog remote sites linked to the telephone network via modem. The modem emulation transmits data according to the V.32bis procedure. This means that data connections of up to 14400 baud can be operated.

New Information and Driver Software for AVM ISDN-Controllers

The latest driver software for your AVM FRITZ!Card USB v2.0 can be found in the Internet at http://www.avm.de. Also available there is the latest information about AVM and our products. Alternatively, the latest controller software can be downloaded from the AVM Data Call Center using Connect32. Just dial + 49 - (0)30 / 39 98 43 00.

Information about the COMMON-ISDN-API (CAPI) 2.0 Interface

Information about the COMMON-ISDN-API interface is available on the Internet at http://www.capi.org.